To configure an interface with a description in Cisco IOS, you would enter interface sub-mode and use the following command:

description IT

Configuring Domain name:

ip domain-name CCNA.net

Configuring password minimum length:

security passwords min-length 10

Configuring user with privilege 15:

username Admin privilege 15 secret Adminpa55

Disabling Domain lookup:

no ip domain-lookup

Encrypting all plaintext passwords:

service password-encryption

Configuring Console timeout (for 10 minutes = 600 seconds)

exec-timeout 10 0

exit

Configuring VTY lines to allow SSH logins:

line vty 0 4

transport input ssh

VTY timeout (for 10 minutes):

exec-timeout 10 0

Using local database for login:

login local

exit

SSH server configuration:

crypto key generate rsa

! When prompted, enter 1024 for key size

1024

Configuring Login banner:

banner login #No Unauthorised Access#

Block login after failed attempts:

login block-for 180 attempts 4 within 30