Packet Tracer Practice Lab

Part 1: Design a VLSM Addressing Scheme

- Addressing Table                                                                                                                               (4 marks)

| Device | Interface |  | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- | --- |
| Police | Fa0/0 | IPv4 |  |  | N/A |
|  | Fa0/1 | IPv4 |  |  | N/A |
|  | E0/0/0 | IPv4 |  |  | N/A |
| PD-1 | VLAN 1 | IPv4 |  |  |  |
| PD-2 | VLAN 1 | IPv4 |  |  |  |
| PD-3 | VLAN 1 | IPv4 |  |  |  |
| PD-1-23 | NIC | IPv4 |  |  |  |
| PD-2-12 | NIC | IPv4 |  |  |  |
| PD-3-11 | NIC | IPv4 |  |  |  |
| You do not need to configure below |  |  |  |  |  |
| Web Server | NIC | IPv4 | 156.62.140.254 | 255.255.255.0 | 156.62.140.1 |

- Objectives

In this lab you will design a VLSM addressing scheme given a network address and host requirements. You will configure addressing on routers, switches, and network hosts.

- Design a VLSM IP addressing scheme given requirements.

- Configure addressing on network devices and hosts.

- Verify IP connectivity.

- Troubleshoot connectivity issues as required.

- Background / Scenario

You have been asked to design, implement, and test an addressing scheme for a customer. The customer has given you the network address that is suitable for the network, the topology, and the host requirements. You will implement and test your design.

- Instructions

You have been given the IPv4 network address 49.50.10.0/24 by your ISP. Your WAN addresses are already configured as show. The host address requirements are:

- Requirements

Host Requirements:

| LAN | Number of Addresses Required |
| --- | --- |
| LAN-Sales | 70 |
| LAN-Design | 56 |
| LAN-Admin | 26 |
| WAN link | 2 |

Design Requirements

- Create the addressing design. Assign the largest subnet first, in descending order. Also, if possible, keep the subnets on the same router contiguous.

- The subnets should be contiguous. There should be no unused address space between subnets.

- Provide the most efficient subnet possible for the point-to-point link between the routers.

- Assign the subnet addresses using the following table, starting with the largest subnet.

IPv4 Network Design Table                                                                                                                          (4 marks)

| LAN | Number of Hosts Needed | Network address/ prefix | First host address | Last host address | Broadcast address |
| --- | --- | --- | --- | --- | --- |
| LAN-Sales | 70 |  |  |  |  |
| LAN-Design | 56 |  |  |  |  |
| LAN-Admin | 26 |  |  |  |  |
| WAN link | 2 |  |  |  |  |

Configuration Requirements

Note: You do not need to configure the WAN and IP side devices.

Step 1: IPv4 Network design

1. Complete the Network Design Table above. Assign the subnet addresses starting from the largest subnet downwards.

2. Complete the Addressing Table on page 1.

Assign the first usable IP address to the router interfaces

Assign the second useable IP address to the switch SVI.

Assign the subsequent addresses to the PCs.

Part 2:PT Implementation

HINT:

You may check your progress and correctness of your work by clicking on Check Results in the PT Activity window, then click on Assessment Items. Click Close to proceed. Ignore the items related to IPv6 and some of the Interface configuration.

Step 1: Router configurations

You must connect a console cable to router from PD-1-23 to configure the router.

1. Basic configurations   (attach screenshot for the configuration)                                     (4 marks)

- Configure hostname: PR

- Configure a motd banner saying No Unauthorized Access

- Configure domain name as CCNA.net

- Configure secure enable password: Ciscoenpa55

- Configure password minimum length 10

- Configure user Admin with privilege 15 and password Adminpa55

- Disable domain lookup

- Enable encryption of all plaintext passwords

- Configure Console password Ciscoconpa55, enable login.

- Configure console line timeout if inactive for 10 mins.

- Configure VTY lines to allow SSH logins.

- Configure timeout if inactive for 10 mins.

- Configure for login using local database

- Configure SSH server with 1024-bit key size

- Configure login banner: No Unauthorised Access

- Configure block for 3 mins after 4 failed attempts with 30 seconds.

2. Interfaces   (attach screenshot for the configuration)

- Configure all the router interfaces with IPv4 addresses

- Configure the interface with descriptions: SALES, DESIGN, and ADMIN, respectively

- Make sure the interfaces are turned on.

Step 2: Switch Configurations  (attach screenshot for the configuration)

You may use the CLI tab in the switch to configure, no need for console cable as for router.

- Configure the SVI (VLAN1 interface) with addresses.

- Configure the default-gateway

- Other basic configurations like for router are normally required but omitted in this exercise.

Step 3: Host configurations

Assign the subsequent addresses to the PCs.

Step 4: Testing   (attach screenshot for the configuration)                                                   (2 mark)

Test using ping from one PC to another.

Step 5:

Submit PT activity results.

Upload your .docx and .pka files here: Lab 10 (Practice Final Lab)

NOTE: Ignore the completion percentage in this exercise. Include screenshots of the configuration within the word file.