# Packet Tracer – Design and Implement a VLSM Addressing Scheme

# Instructions

In this lab you will design a VLSM addressing scheme given a network address and host requirements. You will configure addressing on routers, switches, and network hosts.

- Design a VLSM IP addressing scheme given requirements.

- Configure addressing on network devices and hosts.

- Verify IP connectivity.

- Troubleshoot connectivity issues as required.

1. Design the topology diagram below in packet tracer

Scenario: You have given the network address: 10.1.1.0/24 by your customer. The host requirements are given based on the above topology. Please note that ES-1 LAN requires 11 hosts.

- Arrange the host requirements in a table format from the subnet or network that requires more hosts to the smaller number of hosts.

- Create the addressing design and document your design in a table format.

In IPv4, every subnet mask is 32 bits long. These bits are divided into:

- Network bits (ones): define the network portion.

- Host bits (zeros): define the number of available addresses for hosts.

To calculate the number of usable host addresses in a subnet, we use the formula:

2^n - 2
where n is the number of zeros (host bits) in the subnet mask.

The prefix (CIDR notation) is calculated by subtracting the number of host bits from 32:

Prefix=32−n

Once the prefix is determined, we calculate:

- Network Address: starting point of the subnet.

- First Usable Host: Network Address + 1

- Last Usable Host: Network Address + (2^n - 2)

- Broadcast Address: Last Host + 1

- Next Network Address: Broadcast Address + 1

1. WS2-LAN (47 hosts)

- 2^6−2=62→ n = 6, so prefix = 32 - 6 = /26

- Network Address: 10.1.1.0/26

- First Host: 10.1.1.1

- Last Host: 10.1.1.62

- Broadcast Address: 10.1.1.63

- Next Network Address: 10.1.1.64

2. ES2-LAN (28 hosts)

- 2^5−2=30→ n = 5, so prefix = /27

- Network Address: 10.1.1.64/27

- First Host: 10.1.1.65

- Last Host: 10.1.1.94

- Broadcast Address: 10.1.1.95

- Next Network Address: 10.1.1.96

3. ES1-LAN (11 hosts)

- 2^4−2=14→ n = 4, so prefix = /28

- Network Address: 10.1.1.96/28

- First Host: 10.1.1.97

- Last Host: 10.1.1.110

- Broadcast Address: 10.1.1.111

- Next Network Address: 10.1.1.112

4. WS1-LAN (5 hosts)

- 2^3−2=6→ n = 3, so prefix = /29

- Network Address: 10.1.1.112/29

- First Host: 10.1.1.113

- Last Host: 10.1.1.118

- Broadcast Address: 10.1.1.119

- Next Network Address: 10.1.1.120

5. WAN (2 hosts)

- 2^2−2=2 → n = 2, so prefix = /30

- Network Address: 10.1.1.120/30

- First Host: 10.1.1.121

- Last Host: 10.1.1.122

- Broadcast Address: 10.1.1.123

Note: We do not use broadcast addresses for network configuration.

| Subnet Description | Number of Hosts Needed | Network Address/CIDR | First Usable Host Address | Last Usable Host Address | Broadcast Address |
| --- | --- | --- | --- | --- | --- |
| WS2-LAN | 47 | 10.1.1.0/26 | 10.1.1.1/26 | 10.1.1.62/26 | 10.1.1.63/26 |
| ES2-LAN | 28 | 10.1.1.64/27 | 10.1.1.65/27 | 10.1.1.94/27 | 10.1.1.95/27 |
| ES1-LAN | 11 | 10.1.1.96/28 | 10.1.1.97/28 | 10.1.1.110/28 | 10.1.1.111/28 |
| WS1-LAN | 5 | 10.1.1.112/29 | 10.1.1.113/29 | 10.1.1.118/29 | 10.1.1.119/29 |
| WAN | 2 | 10.1.1.120/30 | 10.1.1.121/30 | 10.1.1.122/30 | 10.1.1.123/30 |

- Configuration requirements:

Note: You will configure addressing on all devices and hosts in the network.

- Assign the first usable IP addresses in the appropriate subnets to [[East]] for the two LAN links and the WAN link.

- Assign the first usable IP addresses in the appropriate subnets to [[West]] for the two LANs links. Assign the last usable IP address for the WAN link.

- Assign the second usable IP addresses in the appropriate subnets to the switches.

- The switch management interface should be reachable from hosts on all of the LANs.

- Assign the last usable IP addresses in the appropriate subnets to the hosts.

- If the addressing design and implementation are correct, all hosts and devices should be reachable over the network.

Hint: you need to create a table for assigning IP addresses to each device and interface as based on the above configuration requirements. The table format is similar to the below table:

| Device | Interface | Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| East | Gig0/0/0 | 10.1.1.97 | 255.255.255.240 | N/A |
| East | Gig0/0/1 | 10.1.1.65 | 255.255.255.224 | N/A |
| East | Se0/1/0 | 10.1.1.121 | 255.255.255.252 | N/A |
| West | Gig0/0/1 | 10.1.1.113 | 255.255.255.248 | N/A |
| West | Gig0/0/0 | 10.1.1.1 | 255.255.255.192 | N/A |
| West | Se0/1/0 | 10.1.1.122 | 255.255.255.252 | N/A |
| ES-1 | VLAN 1 | 10.1.1.98 | 255.255.255.240 | 10.1.1.97 |
| ES-2 | VLAN 1 | 10.1.1.66 | 255.255.255.224 | 10.1.1.65 |
| WS-1 | VLAN 1 | 10.1.1.114 | 255.255.255.248 | 10.1.1.113 |
| WS-2 | VLAN 1 | 10.1.1.2 | 255.255.255.192 | 10.1.1.1 |
| PC E1-22 | NIC | 10.1.1.110 | 255.255.255.240 | 10.1.1.97 |
| PC E2-47 | NIC | 10.1.1.94 | 255.255.255.224 | 10.1.1.65 |
| PC W1-201 | NIC | 10.1.1.118 | 255.255.255.248 | 10.1.1.113 |
| PC W2-87 | NIC | 10.1.1.62 | 255.255.255.192 | 10.1.1.1 |

- Attached screenshots for the configuration of the devices and interfaces.

- Attach screen shots for the “Ping” between the PCs after configuration.